Automatic Headlight Control System Using Simulink and Stateflow

[Your Name], Department of Electronics and Telecommunication Engineering, [Institute Name], Model-Based Design using Simulink & Stateflow, PRN: [Your PRN]

Abstract

This project presents an Automatic Headlight Control System implemented using Simulink and Stateflow for an automotive application. The model senses ambient light intensity, vehicle speed, manual override input, and fault status, then generates the final headlamp command. The complete design is divided into four subsystems: Input Subsystem, Logic Subsystem, Stateflow Controller, and Output Subsystem. The Input Subsystem creates four simulation scenarios: day moving, night moving, tunnel entry, and night stopped. The Logic Subsystem processes raw signals into boolean status signals using relational operators and hysteresis logic. Hysteresis is implemented with two thresholds, 300 lux and 500 lux, to prevent rapid switching of headlights near the decision boundary. The Stateflow Controller uses four states, OFF, ON, HOLD, and MANUAL, to generate the final headlamp command. The Output Subsystem displays, plots, and logs the result for verification. Simulation results confirm correct behavior across all scenarios. The model demonstrates how model-based design can be used to build and validate a practical automotive control system.

Keywords: Automatic headlight control, Simulink, Stateflow, automotive control, hysteresis, model-based design.

| I. Introduction Modern vehicles use embedded control systems to improve safety, automation, and driver convenience. One common example is automatic headlight control, which turns the headlights ON or OFF according to surrounding brightness and vehicle condition. In practical driving, the vehicle may move in daylight, drive at night, enter a tunnel, stop at a signal in darkness, or require direct manual control from the driver. An automatic controller reduces dependency on driver attention and improves response to changing environmental conditions. This project models an Automatic Headlight Control System in Simulink and Stateflow. Simulink is used for subsystem-based signal processing and scenario generation, while Stateflow is used for state-based decision making. The final output is the signal headlamp_cmd, where 0 represents headlights OFF and 1 represents headlights ON. The main contribution of this work is a clear model-based implementation of a state-driven headlight controller with hysteresis and manual override, tested through four representative scenarios. II. Literature Review Model-Based Design is widely used in automotive control development because it supports modular modeling, simulation, early validation, and direct linkage between control design and implementation [1]. Stateflow is particularly suitable for supervisory automotive controllers because it expresses operating modes, transitions, and priority decisions in a clear finite-state form [2], [3]. Automatic headlight control systems rely on ambient-light sensing and decision thresholds to control lamps according to external brightness conditions [4]. Prior automotive lighting studies also show the importance of robust control logic for safety-related vehicle lighting behavior and adaptive lighting functions [5], [6]. | III. Methodology The model is built using four connected subsystems: Input Subsystem, Logic Subsystem, Stateflow Controller, and Output Subsystem. The simulation is configured as a discrete-time model with fixed step size 0.1 s and stop time 40 s. The Input Subsystem generates AmbientLux, VehicleSpeed, ManualOverride, and Fault. Four scenarios are used for testing: day moving, night moving, tunnel entry, and night stopped. The Logic Subsystem converts raw signals to boolean supervisory inputs. AmbientLux < 300 and AmbientLux > 500 form the lower and upper thresholds for hysteresis. A Unit Delay and logical operators implement the lux-status latch. VehicleSpeed > 0 creates speed_status, while ManualOverride and Fault are passed through. The Stateflow chart implements OFF, ON, HOLD, and MANUAL states. Manual override has the highest priority. OFF drives headlamp_cmd = 0, while ON, HOLD, and MANUAL drive headlamp_cmd = 1. |
| --- | --- |

| IV. Results and Discussion Scenario 1 confirms correct OFF behavior in bright daylight with vehicle motion. Scenario 2 confirms correct ON behavior in dark conditions while the vehicle is moving. Scenario 3 verifies transient response to a tunnel event. The output remains LOW before approximately 10 s, goes HIGH between approximately 10 s and 20 s when brightness drops, and returns LOW after approximately 20 s when the vehicle exits the tunnel. Scenario 4 verifies that the HOLD state keeps headlights ON when the vehicle is stopped at night. This confirms the safety-oriented behavior encoded in the state machine. The results demonstrate correct separation between signal conditioning and state-based decision making. The hysteresis structure prevents flicker, while Stateflow provides a readable and deterministic controller implementation. V. Conclusion The Automatic Headlight Control model successfully demonstrates an automotive control application using Simulink and Stateflow. The model senses operating conditions, processes those signals through hysteresis-based logic, makes a state-based decision, and verifies the output through display, scope, and workspace logging. The simulation results confirm correct response in daylight, night driving, tunnel transition, and stopped-at-night conditions. | References [1] MathWorks, "Model-Based Design with Simulink," MathWorks Documentation. [Online]. Available: https://www.mathworks.com/help/simulink/gs/model-based-design.html [2] MathWorks, "Stateflow Documentation," MathWorks Documentation. [Online]. Available: https://www.mathworks.com/help/stateflow/index.html [3] MathWorks, "Design Finite State Machines in Stateflow," MathWorks Documentation. [Online]. Available: https://www.mathworks.com/help/stateflow/gs/get-started-introduction.html [4] K. M. F. Shahriar, "Automatic headlight controlling of vehicle using ambient light sensor based on phototransistor," ScienceOpen Preprints, 2021. [5] B. H. Kim et al., "Design and hardware-in-the-loop simulation of an automatic headlight control system," IFAC Proceedings Volumes, vol. 39, no. 16, pp. 746-751, 2006. [6] Y. L. Chen et al., "New method of automatic control for vehicle headlights," Optik, vol. 157, pp. 718-723, 2018. |
| --- | --- |

TABLE I. SYSTEM INPUTS AND OUTPUTS

| Signal | Role | Meaning |
| --- | --- | --- |
| AmbientLux | Input | Light level (lux) |
| VehicleSpeed | Input | Moving or stopped |
| ManualOverride | Input | Driver forces lamp ON |
| Fault | Input | Supervisory fault input |
| headlamp_cmd | Output | 0 = OFF, 1 = ON |

TABLE II. SCENARIO DEFINITIONS

| Scenario | Condition | Output |
| --- | --- | --- |
| 1 | Day + Moving | 0 |
| 2 | Night + Moving | 1 |
| 3 | Tunnel Entry | 0 -> 1 -> 0 |
| 4 | Night + Stopped | 1 |

[Insert Figure 1. Input Subsystem Screenshot Here]

Figure 1. Input Subsystem

[Insert Figure 2. Logic Subsystem Screenshot Here]

Figure 2. Logic Subsystem

[Insert Figure 3. Stateflow Controller Screenshot Here]

Figure 3. Stateflow Controller

[Insert Figure 4. Output Subsystem Screenshot Here]

Figure 4. Output Subsystem

[Insert Figure 5. Tunnel Entry Simulation Result Screenshot Here]

Figure 5. Tunnel Entry Simulation Result